Acadia University

School of Computer Science

Name :                                                          Section:

ID # :

Course Number Instructor Digital Systems C1213 Ali Elkateeb

Examination Date Time # of pages Final Exam (40%) December 14, 1998 Three hours 7

Not Allowed Material calculator

Special Instructions 1. Closed book exam. 2. Solve all problems. 3. Please do all work on the question sheets. Use the back of the paper if necessary.

Happy Holidays .  Good Luck!

1.

a.   (2 points) perform the binary operation of 70 - 82. Both numbers are in Twos complement form.

b.  (2 points) Add the following two BCD numbers:

3 1 1

+    2 8 9

c.   (1 point)  Convert the following binary number to Hexadecimal:

01010110111011

2. (5 points) Given the following function in sum of products form (not necessary minimized):

F(A,B,C,D) = A'BC + AD + AC

Implement the function F using

(a) An 8:1 multiplexer.

(b) A 4:16 decoder with a 16-input OR gate.

(c) A 16-word ROM.

(d)  A  PLA. (Note: Use the generic form for PLA and select a PLA that would able you to fit the whole design in it).

3.  (5 points) Draw the timing diagram of the following circuit. Assume that the initial values of all flip-flops are at reset state.

D     QC

QCb

D    QB

QBb

D     QA

QAb

Clock

1

QA 0

1

QC 0

clock

1

QB 0

4.

a.  (2 points) Draw the circuit diagram for 3-bit shift left register. Use D flip-flops in your circuit.

b.  (3 points)  Draw the circuit diagram for 3-bit parallel-in parallel-out register. Use JK flip flop in your circuit.

5.   (5 Points) Given the following Boolean functions:

W(A,B,C,D) = A’B’ + C’D’ +ABCD

X(A,B,C,D) = ABD + AC + A’D’

Z(A,B,C,D) = D + C + B + A + AD'

Implement the logic design using  a PAL  circuit (shown below). For each output from the AND array, indicate along the wire the product term it is implementing.

6.  (5 points) Design and implement a  synchronous counter that counts in the following sequence: 111, 101, 110, 011, 100 and repeat. Use JK  flip-flops in your design. Your answer should include the state diagram, the next state transition table, the remapped next state (excitation table), and the final circuit diagram.

7.  (5 points) Derive the truth table for the following digital circuit:

J      QB

K   QBb

clock

+ (logic one)

J        QA

K    QAb

J        QC

K     QCb

OR

Note: Assume all flip-flops are initialized to reset state.

QA	QB	QC	clock

0	  0	  0	Initial Condition

8. (5 points) Implement a combinational logic circuit that converts 4-bit sign and magnitude numbers into corresponding 4-bit ones complement numbers. Draw an input/output conversion truth table, intermediate K-maps, and your minimized two-level logic description.