Objective

The objective of project two is to design a voltage source using the HC11.  This project will expand on the knowledge of on-time and off-time learned in previous experiments.  The project will be designed to take in an input number (decimal) between 0 and 50 and output a signal connected to a simple RC filter.  The main task is to design the system so that the average output voltage (after filtering) is equal to 1/10 the input number.  In addition to this task, the program should be written to display the user-entered voltage on a seven-segment display.

Theory

In theory, the program should monitor the keyboard entries to the terminal in order to change the desired voltage.   After the keyboard entry is made, the seven-segment display, along with the voltage through the RC filter, should change according to the input.

The voltage through the RC filter is changed by using an OC2 interrupt to adjust the on-time and off-time.  As learned in the previous experiment, by adjusting the amount of time that a signal is sent through Port D, the voltage that is seen across Port D is changed corresponding to the input voltage.  The on-time + off-time is kept constant, while the on-time is changed to generate the required average voltage.

There are many design requirements that must be taken into consideration before writing the program.  If the input is less than two digits, the program should assume that the first digit is zero.  If the input has more than two digits, the program should use only the last two digits.  The program should discard illegal inputs and send a beep to the terminal in order to inform the user of the illegal entry.

There are also many design requirements concerning the on-time and off-time to take into consideration.  The on-time and off-time should be selected so that the on-time + off-time is constant.  The product of the values of the resistor and the capacitor should be at least tem times the on-time + off-time.

Results and Conclusions

The simple voltage source was successfully implemented.  The design allowed a user to enter a voltage between 0 and 50. After the user pressed the enter key, the seven-segment display showed the new voltage while the voltage through the RC circuit was changed to match the new voltage.

This project helped to understand OC2 interrupts and what they can be used for.  Not only did the project help to understand interrupts, the project also helped to understand what an HC11 is capable of doing.

Pseudo Code

OUT1BYT	EQU	$FFBB

OUTA		EQU	$FFB8

OUTSTRG	EQU	$FFC7

BIT6		EQU	%01000000

BIT3		EQU	%00001000

BIT2		EQU	%00000100

TOC2		EQU	$1018

TFLG1		EQU	$1023

TMSK1		EQU	$1022

PORTD		EQU	$1008

DDRD		EQU	$1009

INCHAR		EQU	$FFCD

OUTCRLF	EQU	$FFC4

OUTLHLF	EQU	$FFB2

OUTRHLF	EQU	$FFB5

ENTER		EQU	$0D

SPACE		EQU	$20

BEEP		EQU	$07

BACK		EQU	$08

ESCAPE		EQU	#27

ORG	$3000		Originate data at $3000

FLAG		RMB	1		Reserve 1 byte of for FLAG

OFFTIME	RMB	2

ONTIME		RMB	2

INDEX		RMB	1

INDEX1		RMB	1

TEMP		RMB	2

INPUT		RMB	1

LEDNUM	RMB	1

NUMB1		RMB	1

ONTABLE	FDB	$0074,$0190,$0320,$04B0,$0640,$07D0,$0960

FDB	$0AF0,$0C80,$0E10,$0FA0,$1130,$12C0,$1450

FDB	$15E0,$1770,$1900,$1A90,$1C20,$1DB0,$1F40

FDB	$20D0,$2260,$23F0,$2580,$2710,$28A0,$2A30

FDB	$2BC0,$2D50,$2EE0,$3070,$3200,$3390,$3520

FDB	$36B0,$3840,$39D0,$3B60,$3CF0,$3E80,$4010

FDB	$41A0,$4330,$44C0,$4650,$47E0,$4970,$4B00

FDB	$4C90,$4FAC

OFFTBLE	FDB	$4FAC,$4E70,$4CE0,$4B50,$49C0,$4830,$46A0

FDB	$4510,$4380,$41F0,$3060,$3ED0,$3E40,$3BB0

FDB	$3A30,$3890,$3700,$3570,$33E0,$3350,$20C0

FDB	$2F30,$2DA0,$2C10,$2A80,$28F0,$2760,$25D0

FDB	$2440,$22B0,$2120,$1F90,$1E00,$1C70,$1AE0

FDB	$1950,$17C0,$1630,$14A0,$13A0,$1180,$0FF0

FDB	$0E60,$0CD0,$0B40,$09B0,$0820,$0590,$0500

FDB	$0370,$0074

ASCII		FCB	$30,$31,$32,$33,$34,$35,$36

FCB	$37,$38,$39

FCB	$04

INPTBLE	FCB	$00,$01,$02,$03,$04,$05,$06

FCB	$07,$08,$09,$10,$11,$12,$13

FCB	$14,$15,$16,$17,$18,$19,$20

FCB	$21,$22,$23,$24,$25,$26,$27

FCB	$28,$29,$30,$31,$32,$33,$34

FCB	$35,$36,$37,$38,$39,$40,$41

FCB	$42,$43,$44,$45,$46,$47,$48

FCB	$49,$50

ON		FCC	'Ontime = '

FCB	$04

OFF		FCC	'Offtime = '

FCB	$04

NAME		FCC	'Edward Rivera'

FCB	$04

PROMPT	FCC	'Please enter desired value of voltage'

FCB	$04

ERROR		FCC	'Entered voltage too high ( > 50 ), try again'

FCB	$04

VOLT		FCC	'Desired voltage is: '

FCB	$04

ORG	$00DC

JMP	OC2ISR

ORG	$2100		Beginning of Main at $2100

LDX	#NAME		Load X with my name

JSR	OUTSTRG	Print my name to the screen

LDD	#$74		Initializes output voltage to zero

STD	ONTIME		Stores value in ACCD to ONTIME

LDD	#$4FAC		Initializes value for OFFTIME

STD	OFFTIME	Stores value in ACCD to OFFTIME

CLR	FLAG		Clears flag for interrupts

LDAB	#$3F

STAB	DDRD		Sets up output bits of PORTD

SEI			Set interrupt flag

LDAB	#BIT6		Load ACCB with the contents of BIT6

STAB	TMSK1		Store ACCB to TMSK1

CLI			Clear interrupt flag

LOOP		CLR	INPUT		Clear the label INPUT

JSR	OUTCRLF

LDX	#ON		Load X with the address of ON

JSR	OUTSTRG	Print the contents of ON

LDX	#ONTIME	Load X with the address of ONTIME

JSR	OUT1BYT	Print 1st half of ONTIME value

JSR	OUT1BYT	Print 2nd half of ONTIME value

LDX	#OFF		Load X with the address of OFF

JSR	OUTSTRG	Print the contents of OFF

LDX	#OFFTIME	Load X with the address of OFFTIME

JSR	OUT1BYT	Print 1st half of OFFTIME value

JSR	OUT1BYT	Print 2nd half of OFFTIME value

JSR	OUTCRLF

LDX	#PROMPT	Load X with the address of PROMPT

JSR	OUTSTRG	Print prompt to user to enter number

JSR	OUTCRLF

USERINP	CL?ÁY?3?¿33?33?3?3

????333333333??3? ? ?333333333333333?¤3333?¤3333?¤33333333]333333S3S333S333S333S333S3µ33333U333?333?333?333?3

3?3T3U333?3¶3?333?333?333?333?333?333?333?333?33?333?333?333?333?333?3$3?3?3?3p3?3?333333333S333?33333333333?333?333?333?333?33A	#ENTER		Compares keystroke to ENTER key

BEQ	STORE		Branches to STORE if key is ENTER key

CMPA	#SPACE		Compares keystroke to SPACE key

BEQ	STORE		Branches to STORE is key is SPACE key

LDX	#ASCII		Load X with the address of ASCII

GETNUM	CMPA	$00,X		Compares keystroke to X+0

BEQ	FOUNDIT	Branch to FOUNDIT if keystroke is found in ASCII table

INX			Increment X

INC	INDEX		Increment INDEX

LDAB	$00,X		Load ACCB with X+0

CMPB	#$04		Compare ACCB with terminating character

BEQ	UNKNOWN	Branch to UNKNOWN if ACCB has term. char.

BRA	GETNUM	Branch always to GETNUM

DONE1		SWI			If ESCAPE key is hit, quit program

FOUNDIT	LDY	#TEMP		Load Y with the address of TEMP

LDAB	$01,Y		Load ACCB with contents of Y+1

STAB	$00,Y		Store contents of ACCB to Y+0

INY			Increment Y

SUBA	#$30		Change value in ACCA from ASCII to digit needed

STAA	$00,Y		Store contents of ACCB to Y+0

BRA	GETCHAR	Branch always to GETCHAR

UNKNOWN	LDAA	#BACK		Load ACCA with value of BACK

JSR	OUTA		Print backspace on screen

LDAA	#SPACE		Load ACCA with the value of SPACE

JSR	OUTA		Print space on screen

LDAA	#BACK		Load ACCA with value of BACK

JSR	OUTA		Print backspace on screen

LDAA	#BEEP		Load ACCA with the value of BEEP

JSR	OUTA		Beep to user

BRA	GETCHAR	Branch always to GETCHAR

STORE		LDX	#TEMP		Load X with the address of TEMP

LDY	#INPUT		Load Y with the address of INPUT

LDAA	$00,X		Load ACCA with the contents of X+0

STAA	NUMB1		Store first digit of number into NUMB1

LSLA			Left shift the contents of ACCA

LSLA			Left shift the contents of ACCA

LSLA			Left shift the contents of ACCA

LSLA			Left shift the contents of ACCA

ADDA	$01,X		Add ACCA with the contents of X+1

STAA	$00,Y		Store contents of ACCA to Y+1

LDAA	INPUT		Load ACCA with the contents of INPUT

CMPA	#$50		Compare ACCA to the value 50

BGT	RESTART	Branch to RESTART if INPUT > 50

JSR	OUTCRLF

LDX	#VOLT		Load X with the address of VOLT

JSR	OUTSTRG	Print the string contents of VOLT

LDAA	INPUT		Load ACCA with the contents of INPUT

JSR	OUTLHLF	Print the left nibble of INPUT

LDAA	#'.'

JSR	OUTA		Print a decimal on the screen

LDAA	INPUT		Load ACCA with the contents of INPUT

JSR	OUTRHLF	Print the right nibble of INPUT

JSR	LED		Jump to the subroutine LED

CLRA			Clear ACCA

LDX	#INPTBLE	Load X with the address of INPTBLE

LOOPS		LDAB	$00,X		Load ACCB with the contents of X+0

CMPB	INPUT		Compare ACCB with the content of INPUT

BEQ	CONTINUE	Branch to CONTINUE if ACCB contains the value in INPUT

INX			Increment X

INCA			Increment ACCA

CMPA	#51		Compare ACCA with the value 51

BEQ	RESTART	Branch to RESTART if ACCA contains the value 51

BRA	LOOPS		Branch always to LOOPS

CONTINUE	STAA	INPUT		Store the contents of ACCA in INPUT

LDAB	INPUT		Load ACCB with the contents of INPUT

LDX	#ONTABLE	Load X with the address of ONTABLE

LDY	#OFFTBLE	Load Y with the address of OFFTBLE

ABX			Add B to X

ABX			Add B to X

ABY			Add B to Y

ABY			Add B to Y

LDD	$00,X		Load ACCD with the contents of X+0

STD	ONTIME		Store ACCD to ONTIME

LDD	$00,Y		Load ACCD with the contents of Y+0

STD	OFFTIME	Store ACCD to OFFTIME

JMP	LOOP		Jump to LOOP

RESTART	LDAA	#BEEP		Load ACCA with the value of BEEP

JSR	OUTA		Produces a beep

JSR	OUTCRLF

LDX	#ERROR		Load X with the address of ERROR

JSR	OUTSTRG	Print the string contents of ERROR

JSR	OUTCRLF

JMP	USERINP	Jumps to USERINP

LED		LDAB	NUMB1		Load ACCB with the contents of NUMB1

LSLB			Logical left shift ACCB

LSLB			Logical left shift ACCB

LSLB			Logical left shift ACCB

STAB	LEDNUM	Store the contents of ACCB to LEDNUM

RTS

OC2ISR		LDAA	TFLG1		Load ACCA with the contents of TTFLG1

ANDA	#BIT6		AND ACCA with BIT6

BEQ	ISRDONE	Check for false interrupts

LDAB	FLAG		Load ACCB with the contents of FLAG

ANDB	#BIT2		AND ACCB with BIT2

EORB	LEDNUM	Exclusive OR ACCB with LEDNUM

STAB	PORTD		Store ACCB to PORTD

TST	FLAG		Test if FLAG is zero

BEQ	WENTLOW	Branch to WENTLOW if FLAG is zero

LDD	ONTIME	Load ACCD with the contents of ONTIME

ADDD	$1018		Reschedule the next interrupt

STD	$1018

BRA	TOGGLE

WENTLOW	LDD	OFFTIME	Load ACCD with the contents of OFFTIME

ADDD	$1018		Reschedule the next interrupt

STD	$1018

TOGGLE	LDAB	FLAG		Load ACCB with the contents of FLAG

EORB	#BIT2		Exclusive OR ACCB with BIT2

STAB	FLAG		Store ACCB to FLAG

LDAA	#BIT6		Load ACCA with BIT6

STAA	TFLG1		Reset TFLG1

ISRDONE	RTI