# Objective:

Design a simple computer design, which include registers, instructions decode and bus control using the Xilinx Design Interface and the XS40 board.. It must be able to perform a load, clear, shift left or right and swap operations.

Components and its Usage:

For MUX block diagram:

- Four 8:1 MUX – select which register is taking to load.

- One TEMP[3:0] bus line – comes from TEMP register

- One RZ[3:0] bus line – comes from R0 register

- One RONE[3:0] bus line – comes from R1 register

- One OPERAND [3:0] bus line – comes from external input 3 bit data.

- One OUTPUT [3:0] bus line – 3 bit data will go to MAIN [3:0] bus line.

For IR block diagram:

- Three FDC flip flop.

- Three 2:1 MUX – when select line (S0) is 1, it will load IR register.

- One input bus line (input [2:0]) – three bit instruction.

- Input Clock – CLK

- Input Clear – CLR, clear instruction register

- Output I0,I1,I2 – instructions

For four registers (R0, R1, Temp, Output) block diagram:

- Four FDC flip flop.

- Four 2:1 MUX – when select line (load) is 1, it will load register.

- One input bus line (REG [3:0]) – 3-bit data goes in register.

- One output bus line (OUT [3:0]) – 3 bit data comes out register.

- Input Clock – CLK

- Input Clear – CLR, clear register.

For main block diagram:

- One MUX block - select which register is going to load

- Four Registers block.

- One INS [2:0] bus-line for instructions.

- One OPERAND [3:0] bus-line for data.

- One MAIN [3:0] bus-line for data transfer.

- Logic gate for control signal.

- ( S0,S1,S2,LDIR,LDR0,LDR1,LDOR,LDTEMP,RESETSC,CLEAR )

- One 3:8 decoder – for decoding the instruction (IN0, IN1, IN2, IN3, IN4, IN5, IN6, IN7)

- One 2:4 decoder – for decoding the timing (T0, T1, T2, T3)

- One SEG display – display output on FPGA board.

- INBUFs

- OBUFs

- IPADs – use for input pin

- OPADs – use for output pin

For SC block diagram:

- One XOR Gate

- One inverter

- Two FDC flip flop

- Output Q0 and Q1 – it will go to 2:4 decoder to get time (T0, T1, T2, T3)

- Input CLK and CLR – for clear counter

Design Procedure:

- Construct the instruction format as following:

| Description | Binary | Decoder | Operation |
| --- | --- | --- | --- |
| Clear | 000 | IN0 | Clear register |
| Load R0 | 001 | IN1 | R0OPRAND[3:0] |
| Load R1 | 010 | IN2 | R1OPRAND[3:0] |
| Load OR | 011 | IN3 | ORR0 |
|  | 100 | IN4 | ORR1 |
| SHL R0 | 101 | IN5 | R0SHL R0 |
| SHL R1 | 110 | IN6 | R1SHL R1 |
| SWAP | 111 | IN7 | SWAPING R0&R1 |

- Construct the micro-operation table as following:

| Load IR | T0 | IRINS[2:0] |
| --- | --- | --- |
| Clear | IN0 | Rn0 SCSC+1 |
|  | IN0T2 | SC0 |
| Load R0 | IN1T1 | R0OPRAND[3:0]SCSC+1 |
|  | IN1T2 | SC0 |
| Load R1 | IN2T1 | R1OPRAND[3:0]SCSC+1 |
|  | IN2T2 | SC0 |
| Load OR (out put reg.) | IN3T1 | ORR0 SCSC+1 |
|  | IN3T2 | SC0 |
|  | IN4T1 | ORR1 SCSC+1 |
|  | IN4T2 | SC0 |
| SHL R0 | IN5T1 | R0SHLR0 SCSC+1 |
|  | IN5T2 | SC0 |
| SHL R1 | IN6T1 | R1SHLR1 SCSC+1 |
|  | IN6T2 | SC0 |
| SWAP | IN7T1 | TEMPR0 SCSC+1 |
|  | IN7T2 | R0R1 SCSC+1 |
|  | IN7T3 | R1TEMP……END |

Logic for control units:

LDIR T0

LDR0(IN1T1) or (IN5T1) or (IN7T2)

LDR1(IN2T1) or (IN6T1) or (IN7T3)

LDOR(IN3T1) or (IN4T1)

LDTEMP(IN7T1)

RESETSC(IN0 or IN1 or IN2 or IN3 or IN4 or IN5 or IN6) and T2

--- sequence counter clears at T2

ClearIN0

- Finding the equation for each select line:

| S2 S1 S0 | Load from |
| --- | --- |
| 0 0 0 | OPRAND |
| 0 0 1 | R0 |
| 0 1 0 | R1 |
| 0 1 1 | TEMP |
| 1 0 0 | SHL R0 |
| 1 0 1 | SHL R1 |

Logic function for

S0(IN3T1) or (IN7T1) or (IN7T3) or (IN6T1)

---load from R0, temp, SHL R1.

S1(IN4T1) or (IN7T2) or (IN7T3)

---load from R1, temp.

S2(IN5T1) or (IN6T1)

---load from SHL R0, SHL R1.

- Design a sequence counter:

| Present State | Next State | D1 D0 |
| --- | --- | --- |
| 00 | 01 | 00 |
| 01 | 10 | 11 |
| 10 | 11 | 10 |
| 11 | 00 | 01 |

Logic function for sequence counter:

D0(Q0)’

D1(Q0 XOR Q1)

Conclusion:

In the CPU each instruction has an assigned binary code to perform its operation. When the specific operation is completed, sequence counter goes to zero (T0) and another instruction is fetched until all the operations have been completed.