Lab 4:  8-bit processor

ECE 376

TA:  Peter Law

Professor: Ali Elkateeb

Term: Summer 02’

Prepared by:

Brandon Harris

Vladi Gergov

Nick Sitarski

I have neither given nor received aid in the preparation of this report:

Objective:

The objective of this lab was to implement a few new features to lab 3, which was previously completed.  These features are the PC and memory.  This is to be done using Xilinx Design Interface and the XS40 board.

Instruction Format:

We used two formats in our project.  We used Memory reference and Register reference formats.

Memory Reference:

Bit 15 = I

Bit 14-12 = Opcode

Bit 11 – 0 = Address

Register Reference:

Bit 15-12 = Opcode

Bit 11-0 = Register Operation

Computer Instruction Reference Table:

R0->OUT		0000		Sends value in R0 to output register.

R1->OUT		0001		Sends value in R1 to output register.

LSL->R0		0010		Logic shift left value in R0.

LSL->R1		0011		Logic shift left value in R1.

SWAP R0 & R1	0100		Swaps the values in R0 and R1.

MEM->R0		0101		Loads R0 with value from memory.

MEM->R1		0110		Loads R1 with value from memory.

R0->MEM		0111		Loads value from R0 to memory.

R1->MEM		1000		Loads value from R1 to memory.

Micro Operations:

R0->OUT		SC=000 IR=0000		r0_bus <= '1'

SC=001 IR=0000		ld_out <= '1'

SC=010 IR=0000		clr_scntr <= '1'

R1 ->OUT		SC=000 IR=0001		r1_bus <= '1'

SC=001 IR=0001		ld_out <= '1'

SC=010 IR=0001		clr_scntr <= '1'

R0 < LSL R0		SC=000 IR=0010		lsl_r0 <= '1'

SC=001 IR=0010		clr_scntr <= '1'

R1 < LSL R1		SC=000 IR=0011		lsl_r1 <= '1'

SC=001 IR=0011		clr_scntr <= '1'

SWAP R0 & R1	SC=000 IR=0100		swap_r0_r1 <= '1'

SC=001 IR=0100		r0_bus <= '1'

SC=010 IR=0100		ld_temp <= '1'

SC=011 IR=0100		r1_bus <= '1'

SC=100 IR=0100		ld_r0 <= '1'

SC=101 IR=0100		temp_bus <= '1'

SC=110 IR=0100		ld_r1 <= '1'

SC=111 IR=0100		clr_scntr <= '1'

MEM->R0		SC=000 IR=0101		ld_r0 <= '1'

SC=001 IR=0101		mem_bus <= '1'

SC=010 IR=0101		mem_read <= '1'

MEM->R1		SC=000 IR=0110		ld_r1 <= '1'

SC=001 IR=0110		mem_bus <= '1'

SC=010 IR=0110		mem_read <= '1'

R0->MEM		SC=000 IR=0111		r0_bus <= '1'

SC=001 IR=0111		mem_write <= '1'

R1->MEM		SC=000 IR=1000		r1_bus <= '1'

SC=001 IR=1000		mem_write <='1'

Design Details:

In this design, we were supposed to add a program counter, a few new instructions as well as memory.  The new instructions dealt with direct memory addressing mode.  We had to implement instructions that would load values into memory as well as instructions that would read values from memory.  In our design we used VHDL to decode the binary instructions.  In the code, we needed to check if certain values were true.  For example, in order to execute an instruction, we needed to make sure that the sequence counter was in the right position before it would be true.  We also had to design a bus system for the registers.  All of the registers (excluding the MBR) are 4 bit.  That means the outputs are also 4 bits.  We had to implement a series of multiplexers so we could combine the output bits for the registers.  The MAR1 register is used to address memory.  MBR looks at the address value in MAR1 to decide what to access.

For the system reset, we set up an input pin that goes to all of the clr inputs of the registers.  When the input is 1, the values inside of the registers are reset.  When the input is 0, nothing happens.

Conclusion:

This lab was very difficult at first to grasp because everything was new to us.  Once we understood some of the basic concepts everything became clearer however.  This lab would have been much more burdensome if we did not use VHDL.  I regret not using VHDL on lab 3 because it would have made it much simpler.