# Assignment 1:

### ECE 450

Electronic Communications

Spring/Summer 2003

7/29/26

Tatha (Tat) Lohachitkul 		 rattatatat@hotmail.com

Honor Code: I have neither given nor received aid in the preparation of this report.

Tatha (Tat) Lohachitkul

Introduction:

In this assignment we are to do Fourier waveforms (with 8 harmonics) for square and impulse train waveforms for part 1. Part 2, we are to use 100 samples from a Gaussian generator and load them into a spice waveform generator and analyze it.

What is Fourier? It can model any arbitrary signal in terms of since waves.  This is important because linear systems respond to sine waves. By making a complicated signal more simple, superposition can apply the sine waves one by one and individually add the responses. So here is a method of determining the response of a complicated system to a complicated signal.

A square wave is composed of the fundamental plus the odd harmonics and can be represented by this equation:

G(t) = sin(t) + 1/3sin(3t) +1/5sin(5t) +1/7sin(7) + …

An impulse train wave has impulse spikes and they are repeated evenly. It is represented by this equation:

G(t) = 1 + cos(t) + cos(2t) + cos(3t) + …

Results:

The circuits for the square and impulse train waveforms can be found on the next page. It took a couple of hours to figure out where everything goes and how it worked. But finally I was able to produce the waveforms. The square waveform fortunately look like what was expected which had a pattern of square waves. However, I was not able to produce the impulse train waveform. I believe my circuit is correct. I think the problem is somewhere in the simulation. I will figure out it in the next assignment. The Gaussian produced a waveform that had many peaks and lows which was interesting and expected.

Conclusion:

This lab has taught me how to design a circuit for Square and Impulse waveforms. It has given me a chance to become familiar with the different Fourier waveforms. It was also the first time for me to use a sample from a Gaussian generator with the schematic which was very interesting and knowledgeable. In addition I have gotten to use Pspice more productively during this assignment. Overall, this assignment is a great introduction to start of Electronic Communications. It has acquainted me to many new concepts.