Project Design Results of Matlab and Simulink:

Matlab Portion:

%Tan Nguyen, Trang Pham, Nhan Phan

%ECE 460

%WINTER 2003

%Project I

%Part I without a controller D(s)

clc

clear

t=[0:0.01:4];

num=[12.1846];

den=[1 4.127 12.1846];

sys=tf(num,den)

ys=step(sys,t);

plot(t,ys)

title(' Step Response of Part I without the Controller ');

xlabel('Time in second');

ylabel('Magnitude');

grid on

[tp,os,ts]=tposts(t,ys)

Transfer function:

12.18

---------------------

s^2 + 4.127 s + 12.18

tp =

1.1200

os =

0.0999

ts =

1.7000

%Tan Nguyen, Trang Pham, Nhan Phan

%ECE 460

%WINTER 2003

%Project I

%Part I without a controller D(s)

clc

clear

t=[0:0.01:4];

num=[12.12];

den=[1 4.127 12.12];

sys=tf(num,den);

ys=step(sys,t);

pzmap(num,den);

title('The Pole-zero Map of Part I without the Controller');

grid on

[p,z]=pzmap(num,den)

p =

-2.0635 + 2.8039i

-2.0635 - 2.8039i

z =

Empty matrix: 0-by-1

%Tan Nguyen, Trang Pham, Nhan Phan

%ECE 460

%WINTER 2003

%Project I

%Part II with a controller D(s)

clc

clear

t=[0:0.01:2];

num=[80.4184];

den=[1 12.3753 80.4184];

sys=tf(num,den)

ys=step(sys,t);

plot(t,ys)

title(' Step Response of Part II with the Controller D(s)');

xlabel('Time in second');

ylabel('Magnitude');

grid on

[tp,os,ts]=tposts(t,ys)

Transfer function:

80.42

---------------------

s^2 + 12.38 s + 80.42

tp =

0.4800

os =

0.0500

ts =

0.6700

%Tan Nguyen, Trang Pham, Nhan Phan

%ECE 460

%WINTER 2003

%Project I

%Part II with a controller D(s)

clc

clear

t=[0:0.01:2];

num=[80.4184];

den=[1 12.3753 80.4184];

sys=tf(num,den);

pzmap(num,den);

title('The Pole-zero Map of Part II with the Controller D(s)');

grid on

[p,z]=pzmap(num,den)

p =

-6.1876 + 6.4909i

-6.1876 - 6.4909i

z =

Empty matrix: 0-by-1

Simulink Portion: