Context Switch – Process of saving all CP registers to the stack once an interrupt is called

Real Time System – A system that has time constraints that could lead to consequences if not met.

Critical Section – A section in a non reentrant subroutine that can not be interrupted

Reentrant Module – A module that once started by one thread can be interrupted and executed by another thread

Semaphore – A counter that has two atomic functions (wait and signal) apart from initialization that operates on the counter

Real –Time Operating System – A time critical system that manages memory schedules tasks and manages system resources

10/.5 = 20

50/1 = 50

Under worse case conditions 20 inputs can be driven.

10ma

2ma

2ma

2ma

2ma

2ma

construct row

{

unsigned char out;

unsigned char keycode[4];

}

#typedef const struct Row RowType;

RowType ScanTab[5]={

{ 0x70, “abcd”},

{0xB0, “efgh”},

{ 0xD0, “ijkl”},

{ 0xE0, “mnop”},

{ 0x00, “     “}};

void Ritual (void){

PIOC = 0x40;

DDRC = 0xF0;}

Unsigned char Scan(unsigned int *Num){

RowType *pt; unsigned char column, key;

Int j;

(*num)=0; key=0;

pt=&ScanTab[0];

while(pt->out){

PORTC=pt->out;

Column=PORTC;

For (j=3; j>=0; j--){

If((column&0x01)==0){

Key=pt->keycode[j];

(*Num)++;}

column>>=1;}

pt++;}

return key;}

Gadfly – Software loop that check IO status waiting for I/O state

DMA – Interfacing approach that transfers data directly to and from memory

Interrupts – Uses hardware that causes special software execution

Blind Cycle – Software simply waits a fixed amount of time

Periodic Polling – Uses clock interrupt to periodically check I/O status.

Semaphore – a counter it has two atomic functions apart from initialization that operate on a counter.

Que - Stack FIFO

T1       		 	T2   		  		T3   				 T4

10ms			10ms				10ms				10ms

-.5ms context

-1ms interrupt

-.5ms context

10 – 2 =8 ms

per task

6 time through at 40 ms (4*10ms) = 240 ms + 10 ms for T1 + 2 ms left in T2 + 2ms for context switching in T2 = 254 ms