ECE 473 - Embedded Systems

Homework #3

# Solutions

Chapter 2

## Problem 2.1

FUZZY subroutine:

*  Inputs  Th, In 8 bit unsigned integers on stack

*  Output  Out 8 bit unsigned integer in Reg B

Th16	   equ     0  16 bit unsigned Th

In	   equ     4   8 bit unsigned

Th	   equ     5   8 bit unsigned

FUZZY  pshx        Allocate Th16

tsx

ldaa    In,x

cmpa  Th,x

bhs	  Make0   if In>=Th  Out=0

*  If  In<Th	Out = (255*(Th-In))/Th

Calc  	  clra

ldab   Th,x  RegD is 16 bit

std     Th16,x

subb  In,x   RegB=Th-In

ldaa   #255

mul

* RegD=255*(Th-In)

ldx     Th16,x

idiv

xgdx

* RegB=(255*(Th-In))/Th

bra     Done

Make0	  clrb		Out=0

Done	  pulx		Deallocate

rts

# Problem 2.5

Part a)

global RAM		stack RAM		EEPROM

object		0x0000-…		…-0x00FF		0xE000 to 0xFFFF

x1		      xx

x2		      xx

x3									xx

y1					         xx

y2					         xx

z1					         xx

z2					         xx

add3									xx

main									xx

Part b)

data area	   RAM		       stack area	         RAM

x2

x1

y1 of sub

z1

return addr

z2

z3

y2 of main

0,X

2,X

6,X

8,X

not accessible

SP

X

text area            ROM

x3

_add 3 _main

Chapter 3

Problem 3.1

Part a)

data	equ	2	8 bit ASCII character, call by value on stack

Pa	equ	0	relative address of port A

Pb	equ	4	relative address of port B

Start	equ	$10	connected to bit 4

Ack	equ	$01	connected to bit 0

Output	tsy		                 RegY is stack frame pointer

ldaa	data,y		     retrieve data from stack

ldx	#$1000	     RegX points to I/O block

staa	Pb,x		     1) Output data

bset	Pa,x Start 	     2) Pulse Start

bclr	Pa,x Start

Wait	brclr	Pa,x Ack Wait	     3) wait for Ack pulse

rts

Part b)	there are 4 cycles or 0.5μs between when Start is set (the last cycle of the bset instruction), and when it is cleared (the last cycle of the bclr instruction).

Problem 3.3

1) blind cycle: the software simply waits a fixed amount of time & assumes the I/O will

complete.

2) gadfly or busy waiting: checks the I/O status waiting for the done state.

3) interrupt: use hardware to cause software execution.

4) periodic polling: uses a clock interrupt to periodically check the I/O status

5) DMA: transfer data directly to/from memory.

Problem 3.10

Saving context means all registers, except the SP itself, will be saved on the stack.

The PC is incremented so that the address of the next instruction will be stored onto stack ($E011)

Top		CCR		$04

B		$66

A		$55

X (msb)	$AB

X (lsb)		$12

Y (msb)	$CD

Y (lsb)		$34

PC (msb)	$E0

Bottom	PC (lsb)	$11

Chapter 4

Problem 4.1

Part a) Show bit by bit your choice (what and why) for the parallel I/O control register.

| PIOC bit | Value (what?) | Meaning (why?) |
| --- | --- | --- |
| 7 | X | read only bit |
| 6 | 1 | interrupts |
| 5 | X | CWOM affects output only |
| 4 | 1 | Handshake |
| 3 | 0 | Input Handshake |
| 2 | 1 | STRB Pulse |
| 1 | 1 | rising edge of STRA |
| 0 | 1 | positive STRB pulse |

Part b)

void ritual(void)

{unsigned char dummy;

asm("sei"); 				// make ritual atomic

DDRC=0; 			// PortC are inputs

PIOC=0x57;

IRQvector=&IRQhandler; 	// 6811 EVB vector initialization

InitFifo();

dummy=PIOC;		 // Read PIOC part of acknowledge

dummy=PORTCL;		// Read PORTCL part of acknowledge

asm("cli");

}

*assembly solution

STAF	equ	$80		is set on the rise of STRA=STATUS

RIT	sei			Make this atomic

ldaa	#$00

staa	DDRC		PC7-0 inputs

ldaa 	#$57

staa 	PIOC

jsr	InitFifo

ldaa	PIOC		Read PIOC, ReadCL clears STAF

ldaa 	PORTCL

cli			Interrupts occur when STRA rises

rts

Part c)

#pragma interrupt_handler IRQhandler()

void IRQhandler(void)

{unsigned char dummy;

dummy=PIOC;	// Read PIOC part of acknowledge

put(PORTCL);}	// Read PORTCL part of acknowledge

*assembly solution

IRQHAN	ldaa	PIOC		STAF set on rise of STRA

ldaa	PORTC	read data, clears STAF, ACK interrupt

bsr	Put		Save data in FIFO

rti

Problem 4.2

Part a) Show bit by bit your choice (what and why) for the parallel I/O control register.

| PIOC bit | Value (what?) | Meaning (why?) |
| --- | --- | --- |
| 7 | X | STAF is read only |
| 6 | 1 | STAI Interrupt synchronization |
| 5 | 0 | CWOM Normal Totem Pole Outputs |
| 4 | 1 | HNDS, Output handshake |
| 3 | 1 | OIN |
| 2 | 0 | PLS, level mode (not pulse) |
| 1 | 1 | EGA, rise of STRA sets STAF |
| 0 | 1 | INVB, value of STRB after write CL |

Part b)

unsigned char DONE;		// flag set when line output complete

unsigned char*PrintPt;	// Pointer to string

#define disarm HNDS+OIN+EGA+INVB

#define arm STAI+HNDS+OIN+EGA+INVB

void Print(unsigned char*pt){

if(*pt<>0){	// skip if null string

asm("sei");	// make ritual atomic

IRQvector=&PrintHan;	// 6811 EVB vector initialization

DDRC=0xFF;	// PC6-0 outputs connected to printer DATA

PIOC=arm;

PORTCL=*pt;	 // Start output of first character

PrintPt=pt+1;	// first interrupt will start 2nd character

DONE=0;	// initialize global data structures

asm("cli"); }

else

DONE=1;}	// finished if string is empty

#pragma interrupt_handler PrintHan()

void PrintHan(void){unsigned char data;

if((PIOC&STAF)==0)asm("swi");  // illegal interrupt

data=*PrintPt++;	// Next character to print

if(data)

PORTCL=data; // Output and Acknowledge by clearing STAF

else{

DONE++;

PIOC=disarm; }}

Problem 4.3

void wait(unsigned int time)

{ TOC2=TCNT+time;

TFLG1=0x40			// clear OC2F

while((TFLG1&0x40)= =0);		// wait for time delay

}

void ReceiveMode(void)		// place the keyboard system in receive mode

{ asm("sei");				// make ritual atomic

IRQvector=&STRAHan;

DDRC=0x00;			// Inially both are inputs

PIOC=0x60;

// bit 6 STAI=1 arm;

// bit 5 CWOM=1 open collector,

// bit 4 HNDS=0 simple latched mode, STRB not used

// bit 3 OIN=X not applicable when HNDS=0

// bit 2 PLS=X STRB not used

// bit 1 EGA=0 for all STRA means new data ready

// bit 0 INVB=X STRB not used

flag=0;		// 1 byte FIFO empty

asm("cli");

}

#pragma interrupt_handler STRAHan()

void STRAHan(void)

{unsigned int j, unsigned char dummy, frame;

dummy=PIOC; dummy=PORTCL; 		// clear STAF

frame=0;

for(j=0;j<8;j++){

while((PIOC&0x80)= =0);			// wait for high to low Clock

frame=frame>>1;				// bit 0 first

frame=frame|(PORTCL&0x80);}		//shift in Data, clear STAF

while((PIOC&0x80)= =0);			// wait for high to low Clock, parity

dummy=PORTCL;				//clear STAF

while((PIOC&0x80)= =0);			// wait for high to low Clock, stop

dummy=PORTCL;				//clear STAF

if(flag==0){info=frame; flag=1;}

DDRC=0x01;					// Clock is output

PORTC=0x00;	// Clock is low

wait(80);

DDRC=0x00;}

void SendCommand(unsigned char command)}	// send command to the keyboard

unsigned int j, unsigned char frame=command; unsigned char dummy, odd;

PIOC=0x21;		// disable STRA interrupts, set STAF on rise

PORTC=0x81;	// Clock and Data high

DDRC=0x81;		// Clock and Data floating

while((PORTC&0x01==0);	// Waits for Clock to be high

// didn’t use STAF because it may already have been high

PORTC=0x80;	// Set clock low

wait(800);		// and wait for 400 μs

PORTC=0x00;	// Make Data low, while Clock still low

DDRC=0x80;		// Make clock an input, while Data is still an output

dummy=PIOC; dummy=PORTCL;	// clear STAF

odd=0x80  		// calculate odd parity=number of one’s in the 9 bits is odd

for(j=0;j<8;j++){

while((PIOC&0x80)= =0);	// wait for low to high Clock

if(frame&0x01) {

PORTC=0x80;

Odd^=0x80;}	// flip parity bit for each "1" in the frame

else

PORTC=0x00;

frame=frame>>1; }		// bit 0 first

while((PIOC&0x80)= =0);	// wait for 9th low to high Clock

PORTC=odd;			// send odd parity

DDRC=0x00; }		// leave both Clock and Data in input mode

Problem 4.5

Part a) the basic idea is to tie status to an input that can generate interrupts, we will use

STRA. First we use input handshake to request an interrupt on the falling edge interrupt of status. Then we will use rising edge on STRA :

- status is tied to both  STRA to cause an interrupt on the fall and the rise

- ack is tied to PB0 as a simple output

- data is tied to Port C as eight inputs

Part b)

PIOC bit	Value(what?)	Meaning(why?)

1		0		falling edge of STRA

void ritual void{ unsigned char dummy;

asm("sei");		// make ritual atomic

DDRC=0	// PortC are inputs

PIOC=0x00	// falling edge

IRQvector=&IRQhandler;	// 6811 EVB vector initialization

InitFifo();

dummy=PIOC;	// Read PIOC part of acknowledge

dummy=PORTCL;	// Read PORTCL part of acknowledge

PORTB=0x01;	// ack=1

asm("cli"); }

Part d)

#pragma interrupt_handler IRQhandler()

void IRQhandler(void) {unsigned char dummy;

dummy=PIOC;	// Read PIOC part of acknowledge

if(PORTB&0x01){	// falling edge

put(PORTCL);}	// Read PORTCL part of acknowledge

PORTB=0x00	// ack=0

PIOC=0x02;}	// next time rising edge

else { 	// rising edge

dummy=PORTCL;	// Read PORTCL part of acknowledge

PORTB=0x01;	// ack=1

PIOC=0x00; }	// next time falling edge