ECE 473 - Embedded Systems

Homework #1

Solutions

Problem 1.1

This table shows the function of devices for the two types of CPU bus cycles.

|  | CPU read cycle | CPU write cycle |
| --- | --- | --- |
| can drive the address bus | processor | processor |
| can drive the data bus | RAM, ROM and I/O | processor |
| can drive the control bus | processor | processor |
| can receive info from data bus | processor | RAM and I/O |

Problem 1.3

- BSET 3 , x $50

- Effective address is 3+x=$0024

Memory location $0024 is changed from $0F to $5F

PC is changed from $E806 to $E809

# Problem 1.6 (parts a) and b) )

- The instruction is  lsl 1,y

- Effective address is $0021, Memory location $0021 goes from $71 to $E2, PC goes from $E100 to $E103

Problem 1.7

| 6811 | Mode | Addr | Op-code | Operand | Cycles | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ldy #$40 | IMM | none | $18CE | $0040 | 4 | RegY=$0040 |
| ldx $30 | DIR | $0030 | $DE | $30 | 4 | RegX=$A5B6 |
| ldy $2002 | EXT | $2002 | $18FE | $2002 | 6 | RegY=$2233 |
| ldy 2,x | (IND,X) | $2002 | $1AEE | $02 | 6 | RegY=$2233 |
| ldy 2,y | (IND,Y) | $4002 | $18EE | $02 | 6 | RegY=$4243 |
| adca #$30 | IMM | none | $89 | $30 | 2 | RegA=$AA,C=0 |
| adca $30 | DIR | $0030 | $99 | $30 | 3 | RegA=$1F,C=1 |
| aba | INH | none | $1B | none | 2 | RegA=$25,C=1 |
| psha | INH | none | $36 | none | 3 | RegA=>stack-- |
| blo 6 | REL | $0006 | $25 | $03 | 3 | C=1,so PC=$0006 |

| 6812 | Mode | Addr | Op-code | Operand | Cycles | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ldy #$40 | IMM | none | $CD | $0040 | 2 | RegY=$0040 |
| ldx $30 | DIR | $30 | $DE | $30 | 3 | RegX=$A5B6 |
| ldy $2002 | EXT | $2002 | $FD | $2002 | 3 | RegY=$2233 |
| ldy 2,x | IND | $2002 | $ED | $02 | 3 | RegY=$2233 |
| ldy 2,y | IND | $4002 | $ED | $42 | 3 | RegY=$4243 |
| adca #$30 | IMM | none | $89 | $30 | 1 | RegA=$AA,C=0 |
| adca $30 | DIR | $30 | $99 | $30 | 3 | RegA=$1F,C=1 |
| aba | INH | none | $1806 | none | 2 | RegA=$25,C=1 |
| psha | INH | none | $36 | none | 2 | RegA=>stack-- |
| blo 6 | REL | $06 | $25 | $03 | 3 | C=1,so PC=$0006 |

## Problem 1.9

- A direction register specifies whether an I/O pin is input (read only) or output (read and write).

- Direction registers make the pin more flexible, allowing for more uses of the pin. Each application can specify the functionality of the pin for its own purposes.

Supplemental Problem:

a.	fan-out = minimum (IOH/IIH, IOL/IIL)

IOH/ IIH=10/0.5=20

IOL/ IIL=50/1=50

Under Worst case condition fan-out = minimum (20,50)

fan-out=20

b.	Since the 50mA of each device is going into the outputs, not out, so the Icc is unaffected (not of importance) under these conditions.

c.

0.5 mA

Input1

Output

when   high

0.5 mA

Input2

IOH =2.5mA

0.5 mA

Input3

0.5 mA

Input4

0.5 mA

Input5

d.

10 mA

Input1

Output

when  low

10 mA

Input2

IOH =50mA

10 mA

Input3

10 mA

Input4

10 mA

Input5