ECE 473 - Embedded Systems

Homework #2

# Solutions

Chapter 7 Questions:

Problem 1

a.    ASCII "9" : hexadecimal $39: binary  00111001

ASCII "A" : hexadecimal $41: binary 01000001

s  0    1  2     3  4  5      6  7    p   s   s  0     1  2  3  4  5   6  7  p  s

b.   Bandwidth = (number of information bits/frame) / (total number of bits/frame) * baud rate

Bandwidth = (8/11)*9600 = 6981.8 baud

## Problem 2

a.   SCI transmit process

transmit

TxD=0

Wait 16 clock times

end

n=0

Wait 16 clock times

TxD=bn

Wait 16 clock times

n=n+1

TxD=1

Wait 16 clock times

n<=7?

yes

no

TxD=T8

b.   SCI receive process

receive

end

RxD=1?

yes

Wait 8 clock times

RxD=1?

yes

no

n=0

Wait 16 clock times

Bn=RxD

n=n+1

n<=7?

yes

Wait 16 clock times

R8=RxD

Wait 16 clock times

RxD=0?

FE=1

yes

no

no

Problem 3

SPI run faster than SCI because it is a synchronous protocol, both devices operate the same clock that allows higher bandwidth, while SCI is asynchronous protocol, the devices have two separate clocks.

Problem 4

RS485 operates faster than RS232 and over longer distances because it is a balanced differential lines protocol, that uses 2 wires per signal, while RS232 is single ended.

Chapter 8 Questions:

Problem 1

IIL=1 mA

IIH=0.1 mA

R = 5/IIH = 5 /0.0001 = 50 K

Problem 2

Algorithm:

Initialization:

- Disable interrupts

- Set PC7-PC4 as outputs

- Set PC3-PC0 as inputs

- Set PORTC to be open collector (CWOM=1)

- Enable STRA interrupt

- Enable interrupts

IRQ ISR Logic:

- Select a row by setting its corresponding output pin to a low state

- Read columns and check if 0 to see if key pressed

- Store all switches that have been pressed

- GOTO 1

Problem 3

Ic=2A

hfe=1000

Ic=βIb=2A ; Ib=2/1000=2mA

5V-2*0.7V=R*Ib

0.002R=5-1.4=3.6V

R=3.6/0.002=1.8K