ECE 473 - Embedded Systems

Weather Station Homework

Due December 2, 2002

- Using the weather station operating system, draw the flowchart showing the software components involved in scheduling and running a task every 110ms.  Start with the beginning of the OC1 ISR and end when the task begins to run.

- Using the weather station operating system, draw the flowchart showing the software components involved in starting task 0 using an interrupt (the IRQ interrupt in this case).  Start with the beginning of the IRQ ISR and end when task 0 begins to run.

- Using the weather station operating system, draw the flowchart showing the software components involved in starting a task from another task.  Start with the task that initiates another task and end when that task begins to run.

- The following table gives the initial conditions (at time 0) of the tasks in the "weather station" system we looked at in class.

| Task | Description | Scheduled By | Time- to-run | Ready Bit | Task Duration* |
| --- | --- | --- | --- | --- | --- |
| Task 0 | Wind Speed | IRQ | 000000 | Not Ready | 0.5ms |
| Task 1 | Wind Direction | Periodic (110ms) | 000001 | Ready | 0.5ms |
| Task 2 | Wind Speed to CRT | Periodic (150ms) | 000001 | Ready | 0.5ms |
| Task 3 | Wind Direction to CRT | Tasks 1 and 5 | 000000 | Not Ready | 0.5ms |
| Task 4 | Wind Speed LEDs | Periodic (100ms) | 000001 | Ready | 0.5ms |
| Task 5 | Zero Wind Speed | Task 0 | 000001 | Ready | 0.5ms |
| Task 6 | High Speed Reset | Periodic (30ms) | 000001 | Ready | 0.5ms |
| Task 7 | Background |  | 000000 | Ready | n/a |
| XSchedule | O/S Schedule Service |  |  |  | 0.2ms |
| XSuspend | O/S Suspend Service |  |  |  | 0.2ms |
| Dispatcher | O/S Task Dispatcher |  |  |  | 0.3ms |
| OC1 ISR | 10ms clock interrupt |  |  |  | 0.5ms |
| IRQ ISR | Anemometer Interrupt |  |  |  | 0.4ms |

- * Task duration does not include O/S service calls or O/S calls to other O/S utilities.  For example, if a task uses the O/S schedule service, the 0.2ms schedule service time is separate from the 0.5ms "Task Duration".

- Starting with the first OC1 interrupt (at time 10ms, or 000001), complete a table similar to the example below showing everything that happens in the next 20ms of execution (10ms through 30ms).  The table should show where each task, ISR or operating system function starts and stops its execution.

  - Assume 0ms (no time) for all context switches.

  - Assume the first anemometer interrupt comes in 14ms from the start of the table (table starts at 10ms, so this is 24ms in “real” time).

Example Table:

| Task | Duration | System Time |
| --- | --- | --- |
| OC1 ISR | 0.5ms | 10.5ms |
| Dispatcher | 0.3ms | 10.8ms |
| . . . | . . . | . . . |