University of Michigan - Dearborn 
ECE 473 - Embedded Systems

Lab #1 - Assembly vs. C

Due Sept. 30, 2002

Lab Description:

Write a program first in 68HC11/12 assembly language, and then in C, that does the following:

- Perform any needed initialization.

- Part A:  Go into a function that continuously toggles PA4 as quickly as possible.  This function should continue to toggle PA4 until a key is pressed on the keyboard.

- Part B:  After a key is pressed, the program should call another function that first toggles PA4 and then prints out the message "Iteration Number n" where n is the number of times the message has printed.  This function should also continue until a key is pressed on the keyboard.  The function should never stop in the middle of a text string, but should print a complete message before exiting.

Example of what the screen should look like:

Part A: 
Toggling PA4 ... Press any key to continue

Part B: 
Iteration Number 1 
Iteration Number 2 
Iteration Number 3 
Iteration Number 4 
Iteration Number 5 
. . .                                                 {Note:  screen will simply be scrolling by) 
Iteration Number 32,334 
Iteration Number 32,335

Program Complete

Tasks to be done for this lab:

- Write this program in assembly language and execute it on your 68HC11 board.

- Write this program in C and execute it on your 68HC11 board.

- Using a logic analyzer or oscilloscope, measure the period of PA4 for Part A and Part B for the assembly program.

- Using a logic analyzer or oscilloscope, measure the period of PA4 for Part A and Part B for the C program.

- Write a report for the lab containing the following information:

  - Introduction describing the tasks and purpose of the lab.

  - Compare the relative sizes of the machine code generated for the assembly and C programs.  Explain the differences.

  - Compare the four different PA4 periods that you measured.  Explain any differences - especially between the corresponding assembly and C measurements.

  - How could your assembly or C code be made more efficient in terms of size and performance?

  - Source code for your assembly program

  - Source code for your C program

Other things you need to know for this lab:

- When measuring code size, do not use a measurement given to you be Windows or DOS.  These numbers are far larger than your actual code size.  You should get the measurement from your S19 or LST files.

- When waiting for an input character in assembly, do not use Buffalo's INCHAR function.  It will not allow your program to continue if there is not an input ready.

- When printing out characters from assembly, do not use Buffalo routines.  They clear the RDRF flag, which will cause your program to miss keyboard inputs.

- For sample out string and input functions to replace the ones in Buffalo, go to the folder ece473-help-info-for-labs (at the vlt ECE 473 site).