ECE 473

LAB #01 DOCUMENTATION

Olamide Adabonyan

Honor Code - “I have neither received nor given aid for this report”

LAB PROCTOR – Samer Fawaz (samfawas@umd.umich.edu)

Objective(s)

To write a program in both C and Assembly Language that will:

- Go into a function that continuously toggles PA4 as quickly as possible.  This function should continue to toggle PA4 until a key is pressed on the keyboard.

- After a key is pressed, the program should call another function that first toggles PA4 and then prints out the message "Iteration Number n" where n is the number of times the message has printed.  This function should also continue until a key is pressed on the keyboard.  The function should never stop in the middle of a text string, but should print a complete message before exiting.

Some abbreviations used in this documentation:

- ASM: Assmebly

- Main: Main function of program (i.e. where program starts from)

- PA4: Port A, pin 4

- SCSR - SCI status register

- RDRF – Receive data register full flag

Procedure

A code is written in both assembly language and C language to perform the objective stated above.  In both programs, Main prints out a text message to the screen saying “Toggling PA4… Push any key to continue”.  Main stays in a continuous loop toggling PA4 until it gets an input from the keyboard.  This is done by checking the RDRF bit in SCSR to see if it is high, if it is, then there has been an input from the keyboard.  If Main detects a function, it calls another function to obtain the second objective of program.

The second objective is achieved by a function (or subroutine for ASM) called iteration in both the C and the Assembly language codes.  This function loops continuously toggling PA4 and printing out the text “Iteration Number: n” (where n is the number of times it has looped), until it detects an input from the keyboard or it loops 32,335 times.

An Oscilloscope is used to measure the period at which PA4 toggles for both functions, for both the C and Assembly code.

Result(s)

C Code:

Size of .s19 file = 5416 bytes

Period for 1st loop = 50 us (micro secs)

Period for 2nd loop = 18 ms (milli secs)

ASM Code:

Size of .s19 file = 966 bytes

Period for 1st loop = 20 us (micro secs)

Period for 2nd loop = 14 ms (milli secs)

From the above results, ASM code proves to be more efficient because the .s19 file size it produces is smaller that that of the C code.  Also the program written in ASM takes less execution time than that of the program written with C code. The ASM code is hence more efficient.  Also the ASM code does not need compiling (only assembling).

Objective was successfully achieved.  All code and flowchart(s) used for program are in their respective section of this documentation.

Flowchart(s)…

ASM Code...

***************************************************

* Term:		Fall 2003

* Course:	ECE 473

* Name:		Olamide Adabonyan

* E-mail:	o_mide@yahoo.com

*

* REVISION HISTORY

*

* First Draft:  9/19/02

*   -Progress:	-PA4 not toggling correctly

*		-Iteration # printing in Hex form

*	 date:	9/19/02

*		-PA4 issue corrected

*		-Iteration # converted to Dec form

*		-2nd loop not quiting when a key is pushed

*	 date:	9/27/02

*		-subroutine added to read input buffer (nb: buffalo's INPUT does not work for this)

*		-PROGRAM COMPLETE...

*

* Date Submitted: 	9/30/02

* Demo Approval: 	_______

*

* MEMORY MANAGEMENT

*

* Program starts at:	$2000

* Data starts at: 	$3000

* variables: 		$3200

* Page 0 usage: 	Not applicable

*

***************************************************

***************************************************

* EQUATES

***************************************************

* MONITOR EQUATES

INPUT		EQU	$FFAC

OUTSTRG		EQU	$FFC7

OUTCRLF		EQU	$FFC4

OUTRHLF		EQU	$FFB5

OUTA		EQU	$FFB8 	*4 DEBUGING PURPOSES

OUT1BSP		EQU	$FFBE 	*4 DEBUGING PURPOSES

OUT2BSP		EQU	$FFC1 	*4 DEBUGING PURPOSES

* REGS & I/O PORTS EQUATES

REGS		EQU	$1000

PORTA		EQU	$0

SCSR    	EQU     $102E

SCDR    	EQU     $102F

RDRF    	EQU     %00100000

TDRE    	EQU     %10000000

* MEMORY MAP EQUATE EQUATES

PROG		EQU	$2000

DATA		EQU	$3000

VARS		EQU	$3200

* CONSTANT EQUATES

EOT		EQU	$04

ESC		EQU	$1B

ENTER		EQU	$0D

SPACE		EQU	$20

SMALLO		EQU	$6F

CAPO		EQU	$4F

***************************************************

* PRELOADED INFO IN MEMORY / VARIABLE DECLARATION

***************************************************

* LOOKUP TABLES/MESSAGES TO DISPLAY

ORG	DATA

MSGCREDITS	FCC	'Olamide Adabonyan'

FCB	ENTER

FCC	' [ECE 473] | [lab #1-a] | [Sept 30, 2002]'

FCB	ENTER

FCB	EOT

MSG1		FCC	'Toggling PA4...  Press any key to continue'

FCB	ENTER

FCB	EOT

MSG2		FCB	ENTER

FCC	'Program Complete...'

FCB	ENTER

FCB	EOT

MSG3		FCC	'Iteration Number: '

FCB	EOT

MSGTEMP		FCC	' | '

FCB	EOT

* VARIABLES

ORG	VARS

ITERATION_NUM	RMB	2

VAR0		RMB	1

VAR1		RMB	1

VAR2		RMB	1

VAR3		RMB	1

VAR4		RMB	1

***************************************************

* NAME: MAIN

* PURPOSE: - TOGGLE PA4 TILL A KEY IS PUSHED

* ENTRY CONDITIONS: NONE

* EXIT CONDITIONS: NONE

* REGISTERS MODIFIED: ACCA, IX, CCR

***************************************************

ORG	PROG

*init vars/print out msg

LDAA	#$30

STAA	VAR0

STAA	VAR1

STAA	VAR2

STAA	VAR3

STAA	VAR4

LDX	#0

STX	ITERATION_NUM

LDX	#MSG1

JSR	OUTSTRG

LOOP		LDX	#REGS

LDAA	#%00010000

EORA	PORTA,X

STAA	PORTA,X

JSR	INPUT

CMPA	#0

BEQ	LOOP

*call iteration function

LOOP1		JSR	ITERATION

JSR	in_char

CMPA	#0

BNE	ENDPROG

LDD	ITERATION_NUM

CPD	#32335		*change this to small # to test...

BEQ	ENDPROG

BRA	LOOP1

ENDPROG		LDX	#MSG2

JSR	OUTSTRG

LDX	#MSGCREDITS

JSR	OUTSTRG

SWI

***************************************************

* NAME: ITERATION

* PURPOSE: TOGGLES PA4 AND INCREMENTS # OF ITERATION VARIABLES

* ENTRY CONDITIONS: NONE

* EXIT CONDITIONS: NONE

* REGISTERS MODIFIED:

***************************************************

ITERATION	LDX	#REGS

LDAA	#%00010000

EORA	PORTA,X

STAA	PORTA,X

LDX	ITERATION_NUM

INX

STX	ITERATION_NUM

LDAA	VAR0

CMPA	#$39

BEQ	INCVAR1

INC	VAR0

BRA	ENDITERATION

INCVAR1		LDAA	#$30

STAA	VAR0

LDAA	VAR1

CMPA	#$39

BEQ	INCVAR2

INC	VAR1

BRA	ENDITERATION

INCVAR2		LDAA	#$30

STAA	VAR1

LDAA	VAR2

CMPA	#$39

BEQ	INCVAR3

INC	VAR2

BRA	ENDITERATION

INCVAR3		LDAA	#$30

STAA	VAR2

LDAA	VAR3

CMPA	#$39

BEQ	INCVAR4

INC	VAR3

BRA	ENDITERATION

INCVAR4		INC	VAR4

*print out iteration message

ENDITERATION	JSR	OUTCRLF

LDX	#MSG3

JSR	out_str

LDAA	VAR4

JSR	OUTRHLF

LDAA	VAR3

JSR	OUTRHLF

LDAA	VAR2

JSR	OUTRHLF

LDAA	VAR1

JSR	OUTRHLF

LDAA	VAR0

JSR	OUTRHLF

RTS

*********************************************

* out_str

*

* Purpose:  To print a string terminated by an

*   end of transmission character (04) to the

*   SCI port

*

* Inputs:  X register must point to start of string

*

* Outputs: None

*

* Registers Modified:  IX, ACCA

*

*********************************************

out_str

* Wait for SCI transmit data reg to be empty

ldaa    SCSR

anda    #TDRE

beq     out_str

* Get next character of string

next    ldaa    0,x

cmpa    #04       Check for end of string

beq     quit

staa    SCDR      Write character out to SCI data port

* Wait for transmit data reg to be empty

wait    ldaa    SCSR

anda    #TDRE

beq     wait

* Increment pointer to next character and loop

inx

bra     next

quit    rts

*********************************************

* in_char

*

* Purpose:  To read an SCI input from the keyboard

*   if a character is available.  If no character

*   is available, in_char simply returns 0.

*

* Inputs:  None

*

* Outputs: ACCA holds either the ASCII value of the

*   character, or 0 if no character is available.

*

* Registers Modified:  ACCA

*********************************************

in_char

* Check SCI status register to see if a key has been pressed

ldaa    SCSR

anda    #RDRF      Check flag

beq     nochar

ldaa    SCDR       Read the character into ACCA

bra     return

nochar  ldaa    #0         If no character, set ACCA to 0

return  rts

C Code...

/***************************************************
* Term:   Fall 2003
* Course: ECE 473
* Name:	  Olamide Adabonyan
* E-mail: o_mide@yahoo.com
*
* REVISION HISTORY
*
* First Draft: 9/24-26/02
*   		   -while loops not working correctly because inchar()
*					functionstops program
*			 date: 9/27/02
*				   -function to check input buffer coded and running
*				   -PROGRAM COMPLETE...
*
* Date Submitted: 	9/30/02
* Demo Approval: 	________
***************************************************/

#include <hc11.h>

#define endProg() asm("swi")
#define iteration_max 300 //**max # of iterations

char InChar(void);
void iteration(void);

main()
{
 	char input=0, temp;
	printf("Toggling PA4... Press any key to continue\n");

	//RDRF set by default, so it must be cleared [chk pink book 4 instructions]
	temp = SCSR;
	temp = SCDR;
	
	while (input == 0)
	{
		PORTA = PORTA^0x10;
		input = InChar();
	}

	iteration();

	printf("\nProgram Complete...");
	printf("\n\nOlamide Adabonyan");
	printf("\n[ECE 473] | [lab #1-b] | [Sept 30, 2002]");
	endProg();
}


void iteration(void)
{
 	char input2 = 0;
	unsigned short iteration_num=1;
	
	while((iteration_num < (iteration_max + 1)) && (input2 == 0))
	{
		PORTA ^= 0x10;
		printf("\nIteration Number: %d", iteration_num);
		++iteration_num;
		input2 = InChar();
	}
}


char InChar(void)
{
 	char temp;
 	
	if((SCSR & 0x20))
	 temp = SCDR;
	else
	 temp = 0;
					
	return(temp);
}