ECE 473 - Embedded Systems

Lab #2:  Serial and Parallel Interfacing

Due Oct. 28, 2002

Lab Purpose:

- To implement SPI communication between two HC11 boards

- To interface switches, LEDs and motors to an HC11

Administrative Details:

- You may work with one other person on this lab.

- All code must be written in C and demonstrated on an HC11 / HC12 board.

- You must turn in a short lab report as well as your source code.

Task:

As shown in the figure below, the lab will consist of two parts:  HC11 Board A and HC11 Board B.

Hardware Notes:

- The ECE department will provide the motor.  Its Jameco part number is 177498, and the specifications are:  177498 Motor Specifications

- You are responsible for locating and buying any other components that you need, including a general purpose NPN transistor capable of driving the motor, push-button switches, LEDs, general logic, diodes and resistors.

- Make sure your circuit does not require more than about 0.8mA from the HC11 output pin.

- The components you need should be available at most electronic stores.  If you prefer the web, several good places to buy components are:

  - www.digikey.com

  - www.jameco.com

Board A:

This board will be the master in the SPI configuration and will have two push-button switches as inputs.  The outputs will be one green LED and the SPI bus.  Its behavior will be as follows:

- Button 1:  Each time the button is pressed it should generate an IC1 interrupt on HC11 board A.  Board A should then command board B (via SPI) to reduce the motor PWM duty cycle by 5%.

- Button 2:  Each time the button is pressed it should generate an IC2 interrupt on HC11 board A.  Board A should then command board B (via SPI) to increase the motor PWM duty cycle by 5%.

- Green LED:  This should turn on for one second every other second to show that the system is running (i.e. on for 1 second, off for 1 second, on for 1 second, etc.).

Note:  Regardless of how long either button stays depressed, it should only affect the duty cycle by 5%.  The button must be released before it can function again.  (Be careful of switch bouncing!)

Board A's main loop:

- Initialize the system:

  - LED output

  - One second counter, SPI message flags (decrease duty cycle, increase duty cycle)

  - SPI configuration

  - Interrupt flags for RTI, IC1, IC2

  - TCTL2 register (to set input capture edge directions)

  - Interrupt masks for RTI, IC1, IC2

  - other?

- Go into an infinite loop that

  - Checks "decrease duty cycle" flag - If set, it sends the "decrease duty cycle" SPI message

  - Checks "increase duty cycle" flag - If set, it sends the "increase duty cycle" SPI message

Note:  Use gadfly method to wait for SPI messages to complete.

Board A's RTI ISR:

  Checks for "1 second expired" - If so, it toggles the green LED

Board A's IC1 ISR:

- Sets a flag indicating that the "decrease duty cycle" message should be sent

Board A's IC2 ISR:

- Sets a flag indicating that the "increase duty cycle" message should be sent

Board B:

This board will be the slave in the SPI configuration and will have two LEDs (red and green) and a motor as its outputs.  Its behavior will be as follows:

Board A's main loop:

 Initialize the system:

- LED outputs (both off)

- Motor on with PWM duty cycle at 50%

- SPI configuration

- OC1 interrupt flag and mask (for PWM)

 Go into an infinite loop that

- Waits for a SPI message - then increments or decrements the duty cycle by 5%

- Turns on the red LED if the duty cycle was decreased (and turns off green LED)

- Turns on the green LED if the duty cycle was increased (and turns off red LED)

Note:  Use gadfly method to wait for SPI messages.

Board B's OC1 ISR:

 Turn motor output on or off (if it is on, turn it off, etc.)

 Reschedule OC1 for ONTIME (if the motor has been turned on) or OFFTIME (if the motor has been turned off).