Final Exam

Monday, December 18, 2000

Notes:

- Do all your work on the exam answer sheets.  No credit will be given for any work done on this exam paper.

- Turn in both the answer sheets and the exam when you are done.

- Problems may be weighted differently.  Be sure to give enough time to problems with higher point values.

- Show all work.  Partial credit will be given.

# Problem 1 (15 Points):

Assume you are doing RS232 serial transmission at 9600 baud with one start bit (active low), one stop bit (active high), even parity and 8 data bits.

- Draw a timing diagram for transmitting the ASCII value “C” (hex $58).

- For this system, show how to calculate the system bandwidth in bits per second.  You may express your result as a fraction.

# Problem 2 (15 Points):

- List and briefly describe the three events that can cause an SCI interrupt.

- Assuming the SCI transmitter and all SCI interrupts are enabled, give the basic sequence of events (from a programmer’s point of view) involved in transmitting a single SCI character.  Make sure to include any flags that get set along the way.

# Problem 3 (15 Points):

In the weather station operating example we looked at in class, the IRQ ISR is used to schedule Task 0 (the wind speed task) to run as soon as possible.  Let’s say that, instead of starting Task 0, we wanted the IRQ interrupt to start Task 6 as soon as possible.

Starting with the beginning of the IRQ ISR, show the flowchart of high-level software components involved until Task 6 is completed.

# Problem 4 (10 Points):

Assume a chip has the following parameters:  IIL = 100uA, IIH = 10uA, VIH = 4.8V,

VIL = 0.2V.  Show how to connect a switch so that “open” is a logic “0” and “closed” is a logic “1”.  Make sure to minimize power dissipation and to specify any required component values.  You do not need to worry about switch de-bouncing.

# Problem 5 (15 Points):

Six devices need to be connected to a 68HC11.  The devices, their required address space and starting addresses are defined in the table below:

| Device | Address Space | Starting Address |
| --- | --- | --- |
| ROM | 8KB | $E000 |
| RAM | 4KB | $3000 |
| I/O 1 | 8 bytes | $B580 |
| I/O 2 | 16 bytes | $B5C0 |
| I/O 3 | 8 bytes | $B600 |
| I/O 4 | 64 bytes | $B640 |

- Design minimal cost address decoding for this system.  You may show this in either equations or logic gates.

- Assuming the 4KB RAM needs address, data, CS, OE and WE (three control signals are all active low), show the “glue logic” connecting the 68HC11 to the RAM chip.

# Problem 6 (20 Points):

A 4K x 8-bit RAM chip needs to be connected to a 68HC11 running at 2MHz.  Use the attached timing information for the RAM and ‘HC11 to answer the questions.

| CS OE WE | Function |
| --- | --- |
| 0 x 0 | Write |
| 0 0 1 | Read |
| 0 1 1 | Nothing |
| 1 x x | Nothing |

Timing Delays:

- Address Decoding:  30ns

- Address Latch Delay:  20ns

- All other logic gates:  10ns min, 15ns max

- Show a truth table for CS, OE and WE in terms of “select” (output from the address decoder) and the 68HC11 control signals R/W and ECLK.

- Draw a logic diagram (circuit) showing the gates needed to generate these three signals from signals the 68HC11 makes available.  (You may use a “black box” for CS, since you don’t know the actual addresses of the RAM chip, but do show the required inputs to the box.)

- When attached to the ‘HC11, what are the intervals for the RAM chip’s Read Data Available (RDA) and Write Data Required (WDR)?

- Show whether or not this device will be compatible with the 2MHz 68HC11.

# Problem 7 (10 Points):

              - Why is SPI a faster protocol than RS232?  State the difference and explain why it has this affect.

              - Why is RS485 a faster protocol than RS232?  State the difference and explain why it has this affect.

4K x 8-bit RAM Timing

| Parameter | Description | Value |
| --- | --- | --- |
| t1 | Address & CS valid to data valid (max) | 100ns |
| t2 | OE low to data valid (max) | 30ns |
| t3 | Address, CS and OE invalid to data invalid (min) | 0ns |
| t4 | WE pulse width (min) | 80ns |
| t5 | Data setup before WE inactive | 60ns |
| t6 | Data hold after CS, WE, address invalid (min) | 10ns |

###### Key 68HC11 Timing Parameters

Address Available (2MHz):  		AA = (AdV, AdN) = (181, 533)

Read Data Required (2MHz): 	RDR = (450, 510)

Write Data Available (2MHz):	WDA = (378, 533)