Objective:

The objective of this lab is two fold. First, the student will get familiar with Simulink as a tool to simulate discrete-time signals and systems. Second, some typical systems will be simulated and the results will be analyzed using Matlab.

# Exercise #1

- Use Simulink to simulate the system shown in fig. 1. The system is composed of a sampled sine wave of unity amplitude, frequency f1 Hz, and initial phase of /3 radians. The signal will be displayed on the scope. Assume that the sampling frequency (fsamp) is 10 kHz.

- If f1=1.5 KHZ, simulate the system with a time equivalent to 5 periods of the sine wave. Display the waveform on the scope and also export the data of the output to the Matlab workspace to plot the amplitude (discrete-time versus time in msec). Also, plot he amplitude versus the time index, n.

F=1.5KHz

F=11.5KHz

F=8.5KHz

D) For the above cases, give the digital frequencies and indicate if they have aliases in the range 0 < f < 5 KHz.

| Exercise 1 | Analog (2ft) | Digital (2ft) |
| --- | --- | --- |
| Part A | 1.5KHz | 0.3 Rad/sample |
| Part B | 11.5KHz | 2.3 Rad/sample |
| Part C | 8.5KHz | 1.7 Rad/sample |

Both the 11.5KHz and 8.5KHz give digital frequencies, which have aliases in the range of 0< f < 5KHz.

Matlab code

%Part 1A

Fsamp=10000;

Time_1a=output_1a(:,1);

sine_wave=output_1a(:,2);

n_a=Time_1a*Fsamp;

figure(1)

subplot(2,1,1);

stem(Time_1a, sine_wave);

xlabel('time, t(ms)');

ylabel('amplitude');

title('1a-Sine Wave');

grid on;

subplot(2,1,2);

stem(n_a, sine_wave);

xlabel('time index, n(samples)');

ylabel('amplitude');

title('1a-Sine Wave');

grid on;

%Part 1B

Time_1b=output_1b(:,1);

sine_wave=output_1b(:,2);

n_b=Time_1b*Fsamp;

figure(2)

subplot(2,1,1);

stem(Time_1b, sine_wave);

xlabel('time, t(ms)');

ylabel('amplitude');

title('1b-Sine Wave');

grid on;

subplot(2,1,2);

stem(n_b, sine_wave);

xlabel('time index, n(samples)');

ylabel('amplitude');

title('1b-Sine Wave');

grid on;

%Part 1C

Time_1c=output_1c(:,1);

sine_wave=output_1c(:,2);

n_c=Time_1c*Fsamp;

figure(3)

subplot(2,1,1);

stem(Time_1c, sine_wave);

xlabel('time, t(ms)');

ylabel('amplitude');

title('1c-Sine Wave');

grid on;

subplot(2,1,2);

stem(n_c, sine_wave);

xlabel('time index, n(samples)');

ylabel('amplitude');

title('1c-Sine Wave');

grid on;