##### LAB#1:Introduction to Discrete-Time Signal

# Hung Nguyen

Tan Nguyen

###### Minh Le

### Dr. Selim Awad

### Due date: 05/19/03

SPRING/SUMMER 2003

## University of Michigan – Dearborn

HONOR CODE:

We have neither received nor given aid in the presentation of this report.

Signatures:

Objective:

- To familiarize students with Matlab and the basic discrete-time signals.

- Matlab is used as a tool to achieve this purpose.

Procedure/Result(s):

(1) Consider the curve y=4−2x4+x2

What is the range of the variable x, which makes y real? Sketch the curve and estimate the global maximum. Find the value of this maximum and the corresponding value of x at the maximum.

Y = 4−2x4+x2		Y = 4−2x4+x2≥0	   	4+x2≠0		4−2x4+x2≥0

4−2x≥0−2x≥−4x≤2

Therefore, -  x 2

Matlab Code:

x=[-5:0.01:2]';

y=sqrt((4-2.*x)./(4+x.^2));

plot(x,y);

title('Plot of #1');

xlabel('X-Axis');

ylabel('Y-Axis');

grid on

[ymax,indy]=max(y)

xm=x(indy)

ymax=1.0987

indy=418

xm=-0.8300

(2) Plot the following functions on the same graph. Take the interval

0 < t < 30 and choose a suitable .t (e.g., 0.01 s).

y1(t)=1.25e-t

y2(t)=2.02e-0.03t

y3(t)=2.02e-0.3t cos(0.554t+45/90)+1.25e-t

Limit the Y-axis to - 4 and 5 and the X-axis to 0 and 18. Also determine the

following values for y3(t ) at t = 0, t = 5, and y3(t) min.

Matlab Code:

t=[0:0.01:30]';

y1=1.25*exp(-t);

y2=2.02*exp(-0.3*t);

y3=y2.*cos(0.554*t+(45*pi)/90)+y1;

y=[y1,y2,y3];

plot(t,y)

title('Plot of #2');

xlabel('Time in second');

ylabel('Amplitude');

grid on

legend('y1','y2','y3');

v=[0,18,-4,5]

axis(v);

y3_0=y3(t==0)     =1.2500

y3_5=y3(t==5)     =-0.1522

y3_min=min(y3)  = -0.8436

(3) Generate and plot the following sequences:

(a) x1(n) = 0. 8 (n + 5) 			-12 < n < 10

(b) x2(n) =  (n - 335) 			300 < n < 350

(c) x3(n) = 4.5  (n + 4) - 1.5  (n -3) 	-12 < n < 4

(d) x4(n) = u(n) – u(n – 10) 			-5 < n < 20

How many samples do we have for each sequence?

Matlab Code:

%3a)

n=[-12:10]';

x=zeros(size(n));

ind=find(n==-5);

x(ind)=0.8

stem(n,x);

num_samples=length(x)

title('Lab1-Plot#3a')

xlabel('Time-index,n');

ylabel('Amplitude');

grid on

num_samples =23

%3b

n=[300:350]';

x=zeros(size(n));

ind=find(n==335);

x(ind)=1

stem(n,x);

num_samples=length(x)

title('Lab1-Plot#3b')

xlabel('Time-index,n');

ylabel('Amplitude');

grid on

num_samples =51

%3c

n=[-12:4]';

x1=zeros(size(n));

x2=zeros(size(n));

ind1=find(n==-4);

ind2=find(n==3);

x1(ind1)=4.5;

x2(ind2)=-1.5;

x=x1+x2;

stem(n,x)

num_samples=length(x)

title('Lab1-Plot #3c')

xlabel('Time-index,n');

ylabel('Amplitude');

grid on

num_samples = 17

%3d

n=[-5:20]';

x=zeros(size(n));

ind=find(n>=0 & n<=9);

x(ind)=1;

stem(n,x)

num_samples=length(x)

title('Lab1-Plot #3d');

xlabel('Time-index,n');

ylabel('Amplitude');

grid on

num_samples = 26

(4) Plot each of the following sequences. Determine which of them is

periodic. Find the period for the periodic sequences. For the sequence of

part (c) give a simpler formula.

(a) x1(n) = sin (2π14n+π3)			-20 < n < 70

(b) x2(n) = sin (π15n+π3)			-60 < n < 60

(c) x3(n) = sin (3n - /2) 			-10 < n < 10

(d) x4(n) = cos (2π68n)			-10 < n < 50

(e) x5(n) = cos ( π3n)+2sin(π4n+π4)	-40 < n < 40

(f) x5(n) = 2 cos ( π3n3)*sin(π4n+π4)	-40 < n < 40

Matlab Code:

%#4a

na=[-20:70]';

wa=(2*pi)/14;

for i=1:length(na)

ya(i)=sin(wa*(i)+pi/3);

end

subplot(6,1,1)

stem(na,ya);

title('Lab1-Plot#4a');

xlabel('X-axis');

ylabel('Y-axis');

grid on

%#4b

nb=[-60:60]';

wb=pi/15;

for i=1:length(nb)

yb(i)=sin(wb*(i)-pi/3);

end

subplot(6,1,2)

stem(nb,yb);

title('Lab1-Plot#4b');

xlabel('X-axis');

ylabel('Y-axis');

grid on

%#4c

nc=[-10:10]';

wc=3*pi;

for i=1:length(nc)

yc(i)=sin(wc*(i)-pi/2);

end

subplot(6,1,3)

stem(nc,yc);

title('Lab1-Plot#4c');

xlabel('X-axis');

ylabel('Y-axis');

grid on

%4d

nd=[-10:50]';

wd=(2*pi)/sqrt(68);

for i=1:length(nd)

yd(i)=cos(wd*(i));

end

subplot(6,1,4)

stem(nd,yd);

title('Lab1-Plot#4d');

xlabel('X-axis');

ylabel('Y-axis');

grid on

%4e

ne=[-40:40]';

we1=pi/3;

we2=pi/4;

for i=1:length(ne)

ye(i)=cos(we1*(i))+2*sin(we2*(i)+we2);

end

subplot(6,1,5)

stem(ne,ye);

title('Lab1-Plot#4e');

xlabel('X-axis');

ylabel('Y-axis');

grid on

%4f

nf=[-40:40]';

wf1=pi/3;

wf2=pi/4;

for i=1:length(nf)

yf(i)=2*cos(wf1*(i)).*sin(wf2*(i)+wf2);

end

subplot(6,1,6)

stem(nf,yf);

title('Lab1-Plot#4e');

xlabel('X-axis');

ylabel('Y-axis');

grid on

Answer: The period of each sequence is:

a) w12π=114; This is a rational number therefore, x1(n) is a periodic with a period of 14

b) w22π=130; This is a rational number therefore, x2(n) is a periodic with a period of 30

c) This sequence has a  simpler form: sin (3π−π2)=sin(πn+2πn−π2)=−cos(πn)

w32π=32; This is a rational number therefore, x3(n) is periodic with a period of 2

d) w42π=168; This is Not a rational number therefore, x4(n) is not periodic

e) w5a2π=16and w5b2π=18; It is a rational number therefore, x5(n) is periodic with a period of 34 (the least common multiple of the period of the two frequencies).

(5) Generate a sampled sinusoid with the following data:

Signal frequency 0.6 kHz, sampling freq. = 8 kHz,

Initial phase = 30 deg.,

starting time = 0 sec.,

Amplitude = 2,

Ending time = 14 msec.

Make two plots of the resulting signal: one as a function of time (in msec),

and the other as a function of the sample Index n used in tn = nt. Determine

the length of the resulting discrete-time signal and the number of periods of

the sinusoid included in the signal vector.

Matlab Code:

%#5

f=0.6e3;

fsamp=8e3;

T=1/fsamp;

theta=30*pi/180;

time=[0:T/10:14e-3]';

amp=2;

xa=amp*sin(2*pi*f*time+theta);

subplot(2,1,1)

plot(time*1e3,xa,'r')

title('Lab1-Plot of #5');

xlabel('Time in msec');

ylabel('Amplitude');

grid on

tsamp=[0:T:14e-3]';

xb=amp*sin(2*pi*f*tsamp+theta);

n=tsamp/T;

subplot(2,1,2)

stem(n,xb,'b');

xlabel('Sample Index,n');

ylabel('Amplitude');

grid on

N=max(tsamp)/T

period=1/f

num_periods=max(time)/period

N =  112

period =  0.0017

num_periods =  8.4000

(6) Plot the discrete-time exponential signal

x(n) = 2(-0.6)n over the range n = 0,1,2,...,19

Find the sum of the sequence. Verify using the closed form:

Σn=0L−1an=1−aL1−afor a  1

%6

n=[0:18]';

x=2*(-0.6).^n;

stem(n,x);

title('Lab1-Plot of #6');

xlabel('Time Index');

ylabel('Amplitude');

grid on

xm=sum(x)

xm =  1.2501

Σn=0L−1an=1−aL1−a= 21−a201−a=21−(−0.6)191−(−0.6)=1.25008

This is identical to the Matlab result.

(7) Consider the difference equation given as follows:

y(n) = 0.25 y(n-1) + x(n), n > 0 and y(n) = 0 for n < 0.

Find and plot y(n) for the following cases:

(a) x(n) = ä(n)

(b) x(n) = u(n-2)

(c) x(n) = u(n) – u(n – 4)

%7a

n=[-5:10]';

x=zeros(size(n));

ind=find(n>=0);

x(ind)=1;

subplot(2,1,1)

stem(n,x);

title('Lab1-Plot of #7a');

xlabel('n');

ylabel('Amplitude of x');

grid on

y=zeros(size(n));

indnz=ind(1);

for i=indnz:length(x)

y(i)=0.25*y(i-1)+x(i);

end

subplot(2,1,2)

stem(n,y,'r');

title('Impulse response');

xlabel('n');

ylabel('Amplitude of y');

grid on

%7b

n=[-5:10]';

x=zeros(size(n));

ind=find(n==2);

x(ind)=1;

subplot(2,1,1)

stem(n,x);

title('Lab1-Plot of #7b');

xlabel('n');

ylabel('Amplitude of x');

grid on

y=zeros(size(n));

indnz=ind(1);

for i=indnz:length(x)

y(i)=0.25*y(i-1)+x(i);

end

subplot(2,1,2)

stem(n,y,'r');

title('Unit Step Response');

xlabel('n');

ylabel('Amplitude of y');

grid on

%7c

n=[-5:10]';

x=zeros(size(n));

ind=find(n>=0 & n<=4);

x(ind)=1;

subplot(2,1,1)

stem(n,x,'r');

title('Lab1-Plot of #7c');

xlabel('n');

ylabel('Amplitude of x');

grid on

y=zeros(size(n));

indnz=ind(1);

for i=indnz:length(x)

y(i)=0.25*y(i-1)+x(i);

end

subplot(2,1,2)

stem(n,y);

title('Resposne to Unit Step Difference');

xlabel('n');

ylabel('Amplitude of y');

grid on

(8) For the same difference equation given as of part (7), namely

y(n) = 0.25 y(n-1) + x(n), n > 0, find y(n) for the following

conditions:

• y(-1) = 4 (initial condition)

• x(n) = u(n).

%8

clc

clear

n=[-10:15]';

x=zeros(size(n));

ind=find(n>=0);

x(ind)=4;

subplot(2,1,1)

stem(n,x);

title('Lab1-Plot of #8 Unit Step Input');

xlabel('n');

ylabel('Amplitude of x');

grid on

y=zeros(size(n));

index_int_cond=find(n==-1);

y(index_int_cond)=4;

indnz=ind(1);

for i=indnz:length(x)

y(i)=0.25*y(i-1)+x(i);

end

subplot(2,1,2)

stem(n,y,'r');

title('Unit Step Response with Initial Condition of y(-1) =4');

xlabel('n');

ylabel('Amplitude of y');

grid on