ECE 480

Fall 2003

COMPUTER-BASED ASSIGNMENT # 4

Introduction to the simulation of signals and systems

Objective:

The objective of this lab is two fold. First, the student will get familiar with Simulink as a tool to simulate discrete-time and continuous-time signals and systems. Second, some typical systems will be simulatedand the results will be analyzed using Matlab.

Exercise #1

Use Simulink to simulate the system shown in Fig. 1 The system is composed of a sampled sine wave of unity amplitude, frequency f1 Hz, and initial phase shift of p/3 radians. The signal will be displayed on the scope. Assume that the sampling frequency (fsamp) is 10 kHz.

Fig. 1

a) If f1 = 1.5 kHz, simulated the system with a time equivalent to 5 periods of the sine wave. Display the waveform on the scope and also export the data of the output to the Matlab workspace to plot the amplitude (discrete-time versus time in msec). Also, plot the amplitude versus the time index, n.

b) Change the f1 to 11.5 kHz and investigate the output signal. Compare to part a) and comment.

c) What is the apparent frequency of the output if f1 = 8.5 kHz?. Compare to part a) and

comment.

d) For the above cases, give the digital frequencies and indicate if they have aliases in the range 0 < f < 5 kHz.

Exercise #2

Consider the digital filter given by the:

y(n) = 1.8 cos(p/16) y(n-l) - 0.81 y(n-2) + x(n) + 0.5 x(n-1)

a) Using Simulink, assuming fsamp =10 kHz, and a simulation time of 2 msec, find the step response.

b) Determine the steady-state value, Go. Hence, plot the transient response, ytran. Hint: use the model shown in Fig. 2

c) If an input is a pulse equivalent to x(n) = u(n) – u(n-14), find the pulse duration in msec, and hence determine the pulse response. Hint: use the model shown in Fig. 3

d) For a input which is a sine wave of unity amplitude, frequency 1.4 kHz and phase of p/3, as in part b), find the transient and steady-state.

Fig. 2

Fig. 3

Exercise #3

It is required to design an anti-aliasing lowpass filter to filter out frequencies in the frequency band f > 5 kHz. The requirements are as follows:

The stop-band frequency, fs = 5 kHz and (stop-band attenuation) as = 40 dB.

The passband attenuation, ap = 1 dB

Order of filter, n, cannot exceed 7.

a) Choose the approximation that gives the best results.

b) Determine the passband frequency fp.

c) Find the transfer function and hence plot the magnitude and phase responses of your design.

d) For your design, what will be the order of the filter you designed if Butterworth, Chebychev or Elliptic filter approximations are used?

e) In order to test the anti-aliasing lowpass filter, construct the model shown in Fig. 4, which represents a part of a data acquisition system. Assume that the digital filter is given by the following difference equation:

y(n) = 0.53353 y(n-1) -0.7265 y(n-2) + 0.136728{ x(n) - x(n-2)}.

What is the type of this digital filter? Test for stability. Plot the magnitude and phase responses.

f) Simulate the model using Simulink for input frequencies f = 1 kHz, 2, KHz, 4 kHz, 6 kHz and 8 kHz. Verify that there is no aliasing. Investigate the different waveforms seen on the screen of the scopes and comment.

Fig. 4

Exercise #4

In this exercise we would like to model an A/D converter with the following parameters:

- Number of bits is nb

- Full scale-level isVref = 5 volts (this means that the maximum range is + Vref)

- Sampling frequency of 10 kHz.

The model is shown in Fig. 5.

Fig. 5

a) For the following values values of the parameters model the A/D converter and plot the transfer charact\ertics. Hint: Use one period of a low frequency signal (1 Hz).

- nb = 3 bits

- nb = 8 bits.

b) For an analog input sine wave of frequency 10 Hz and amplitude = Vref – (quantization step), plot the output and the quantization noise signals for both cases of part a).

c) Calculate the quantization average power and hence the S/N ratio of the output signals.

Hint: Use about 5 cycles of the sine wave in order to calculate the average power.