##### Final Examination Extra Credit

# Tan Nguyen

### Dr. Selim Awad

### Due date: 08/12/03

SPRING/SUMMER 2003

## University of Michigan – Dearborn

HONOR CODE:

I have neither received nor given aid in the presentation of extra credit.

Signatures:

#1)

- We need a 2nd order Band Pass Filter (BPF)

Digital Filter Specification:

fo=fl∗fu=36∗56=43.68Hz , BW=2π(fu−fl)fs=2π(53−36)200=0.17π

Center  digital Frequency ⇒wo=2πfofs=2π∗43.68200=0.4368π

Now we need to find alpha and beta:

β=cos(wo)=0.197, 2α1+α2=cos(wo)=0.860742 by using quadratic equation we find that α=1.75319, 0.57039, now find the poles for each α. Take one which gives the pole inside the unit circle ⇒α=0.57039.

Find the poles and prove of stability:

Assume that the poles are complex ⇒Zp∧Zp from the 2nd order of BPF transfer function, the poles are the roots of  1-β(1+α)Z−1+αZ−2=0

⇒	(Z-Zp)(Z-Zp*). Therefore, Zp * Zp*=|Zp|^2 =α

⇒	|Zp|<1 implies to be inside the unit circle ⇒α<1 for stability.

⇒Thus , we take α=0.57039

#1 b & c

Matlab Code:

%Tan Nguyen

%#1

%BPF second order IIR

w0=0.4368*pi; %digital center frequency

b=cos(w0);	%the parameter called beta+-+

bw=0.17*pi;	%the 3-dB bandwidth

%Find a (the parameter called alpha)

%sovle the quadratic equation

%(a*a +1)cos(bw)-2a=0

v=[cos(bw),-2, cos(bw)];

%coefficient of quad. equ.

a=roots(v);

%choosing the roots with mag. <1

if abs(a(1))<1

a=a(1);

else

a=a(2);

end

num=(1-a)*[1,0,-1]; %num vec. of transfer function

den=2*[1,-b*(1+a),a];	%den vec. of transfer function

figure(1);

freqz(num,den)

%Finding the difference equation using matlab

[Bdigital,Adigital]=bilinear(num,den,1)

Hz=tf(Bdigital,Adigital,1)

From matlab results, we have the 2nd order BPF that meet the requirement of wo=0.4368 and the BW=0.17

Bdigital =

0.1631   -0.5437    0.1631

Adigital =

1.0000   -1.7361    1.3136

Transfer function:

0.1631 z^2 - 0.5437 z + 0.1631

------------------------------

z^2 - 1.736 z + 1.314

Therefore, the difference equation of the digital filter is as follow:

Y(n)-1.7361y(n-1)+1.3136y(n-2)=0.1631x(n)-0.5437x(n-1)+0.1631x(n-2)

#2

To answer part  ( c ) which is the counter part of Ha(s) such that corresponding digital frequency of 1 is w1=0.5. This means that we have to find the arbitrary T .

Calculation for arbitrary T as follow:

By Zooming Phase response of analog filter we see that, at the phase response of –90 degrees of analog filter /2=1.57. The frequency 1=0.25.

Now using 1=a=0.25,

a=2Ttan(w12),T=20.25πtan(0.5π2)=2.546≈2.55

Therefore, we have the arbitrary value of T=2.55.

Matlab:

N1 = [1 -5.792 9.14];

N2= [1 -4.2 11.487];

Nanalog = conv(N1,N2);

D1 = [1 5.792 9.14];

D2= [1 4.2 11.487];

Danalog = conv(D1,D2);

%Analog Transfer Function

H_s = tf (Nanalog,Danalog)

w=[0:pi/1024:pi];

HS=freqs(Nanalog,Danalog,w);

MagHS=abs(HS);

PhaseHS=angle(HS);

%Prewarping and warping

T=2.55;

%Design of Digital Filter using the Bilinear Trans.

%Finding the coefficient of DE of digital filter

[Ndigital,Ddigital]=bilinear(Nanalog,Danalog,1/T)

Hz=freqz(Ndigital,Ddigital,w);

%Finding the transfer function of digital filter

HDZ=tf(Ndigital,Ddigital,1)

MagHz=abs(Hz);

PhaseHz=angle(Hz);

%plot for each

figure(1)

subplot (2,1,1);

plot (w/pi, MagHS);

ylabel('Magnitude in dB');

xlabel('omega');

title('Magnitude  Frequency response of analog filter');

grid on;

subplot (2,1,2);

plot (w/pi, MagHz);

ylabel('Magnitude in dB');

xlabel('omega');

title('Magnitude Prequency response of digital filter');

grid on;

figure(2)

subplot (2,1,1);

plot (w/pi, PhaseHS, 'r');

xlabel('omega');

ylabel('Angle in Radians');

title('Phase response of analog filter');

grid on;

axis([0,1.2,-4,4]);

subplot (2,1,2);

plot (w/pi, PhaseHz, 'r');

xlabel('omega');

ylabel('Angle in Radians');

title('Phase response of digital filter');

grid on;

axis([0,1.2,-4,4]);

Transfer function:

s^4 - 9.992 s^3 + 44.95 s^2 - 104.9 s + 105

-----------------------------------------------------

s^4 + 9.992 s^3 + 44.95 s^2 + 104.9 s + 105

This is the analog transfer function resulting from Matlab.

From matlab we have the following transfer function and difference equation of the digital filter:

Ndigital =

0.2086    1.1970    2.6207    2.6047    1.0000

Ddigital =

1.0000    2.6047    2.6207    1.1970    0.2086

Transfer function:

0.2086 z^4 + 1.197 z^3 + 2.621 z^2 + 2.605 z + 1

------------------------------------------------------------

z^4 + 2.605 z^3 + 2.621 z^2 + 1.197 z + 0.2086

Therefore,

Y(n)+2.60471y(n-1)+2.6207y(n-2)+1.1970y(n-3)+0.2086y(n-4)=0.2086x(n)+1.1970x(n-1)+2.6207x(n-2)+2.6047x(n-3)+x(n-4)

===============================================================

#3)

a)

y(n)=-0.5y(n-1)+4x(n)

Y(Z)=-0.5z^-1Y(z)+4X(Z) H(Z)=41+0.5z−1

For a input x(n)=nu(n)

X(Z)=-Zd(11−z−1)dzX(Z)=Z(Z−1)2

Y(Z)=X(Z)H(Z)=ZZ+0.5 Z(Z−1)2=4Z2(Z+0.5)(Z−1)2

Y(Z)Z=4Z(Z+0.5)(Z−1)2=A(Z−1)2+B(Z−1)+C(Z+0.5)

A=4Z(Z+0.5)|Z-1=, B=ddz4Z(Z+0.5)|Z=1=98, C=4Z(Z−1)2|Z=−0.5=−98

Y(Z)/Z=83(Z−1)2+98(Z−1)+−98(Z+0.5)

y(n)=83nu(n)+98u(n)−98(−0.5)nu(n)

b)

Yss(n)=98u(n), Ytr(n)=83nu(n)−98(−0.5)nu(n)

c)Verify on matlab

Matlab:

%#3

b=[4];

a=[1, 0.5];

n=[0:150]';	%a column vectors

H=tf(b,a)

x=n;

%set location of ramp

index=find(n <0);

x(index)=0;

y=filter(b,a,x);

%Finding the steady state and transient

yss=y(end)

ytr=y-yss;

figure(1);

subplot(2,1,1)

plot(n,x,'b');

xlabel('Time-index, n');

ylabel('Amplitude');

title('X(n),Unit Ramp Input');

grid on

subplot(2,1,2)

plot(n,y,'r');

xlabel('n');

ylabel('Amplitude');

title('Y(n)-Ramp Response');

grid on

figure(2);

subplot(2,1,1);

plot(n,yss,'x');

xlabel('n');

ylabel('Amplitude');

title(' Steady state response, yss')

grid on;

subplot(2,1,2)

plot(n,ytr);

xlabel('n');

ylabel('Amplitude');

title(' Transient response, ytr')

grid on;

%general form-finding the boundary of transient/steady state response

Nss=0; stop=0; test_plus=0.01; test_minus=-0.01;

for i=1:length(ytr)-1

if test_minus<=ytr(i+1) & ytr(i+1)<=test_plus & stop==0

Nss=i;

stop=1;

elseif (test_plus<ytr(i+1)) | (test_minus>ytr(i+1)) & stop==1

Nss=0;

stop=0;

end

end

Nss